Gain , Noise and Power Study using Cascode Current Mirrors

A Project Based Learning Report Submitted in partial fulfilment of the requirements for the award of the degree

of

Bachelor of Technology

in The Department of ECE

ASIC AND FPGA(23EC2237F), ANALOG VLSI(23VLS3101A), INTEGRATED SYSTEM DESIGN AUTOMATION(23SDEC10A)

Submitted by

2310040060: M. Sowmya

2310040091: N. Srija

2310040112: K. Sushma

2310049143: B. Srija

Under the guidance of

Dr. Ngangbam Phalguni Singh and Dr. Vijay Rao Kumbhare

Department of Electronics and Communication Engineering

Koneru Lakshmaiah Education Foundation, Aziz Nagar

Aziz Nagar – 500075

SEP - 2025

## Chapter 1: Introduction

1.1 General Background

Electronics has become the backbone of modern technology, and integrated circuits (ICs) play a vital role in shaping the functionality of systems we use daily. From smartphones and medical devices to communication networks and industrial automation, ICs ensure compactness, reliability, and efficiency. Among the fundamental blocks of analog ICs are current mirrors, which replicate a reference current into other branches of a circuit. This ability to control and distribute current accurately makes them essential in biasing networks, amplifiers, voltage references, and signal-processing units.

While simple in concept, current mirrors are indispensable because they allow designers to generate stable currents across multiple parts of a circuit without using bulky resistors or external components. Their importance is highlighted in operational amplifiers, filters, regulators, and communication ICs, where precision biasing is required for stability. The study of current mirror topologies, therefore, becomes critical not only for academic understanding but also for practical design applications in VLSI systems.

1.2 Importance of Current Mirrors

The role of current mirrors in analog design goes beyond biasing. They help maintain constant performance across varying conditions, enabling circuits to function predictably. In high-gain amplifiers, current mirrors serve as active loads, improving gain and linearity. In filters, they ensure accurate current replication, which directly impacts filter behavior. Their significance spans data converters, oscillators, and communication systems, making them a core element in virtually every analog design.

Moreover, current mirrors are crucial for enabling scalability in VLSI design. As ICs integrate more transistors into smaller areas, compact and reliable current sources are required. Current mirrors provide this capability while maintaining design simplicity. Their widespread use makes improving their accuracy, efficiency, and stability an ongoing focus of analog circuit research.

1.3 Limitations of Simple Current Mirrors

Despite their usefulness, simple current mirrors suffer from several drawbacks. The most critical is low output resistance, which causes the output current to vary with output voltage changes. This makes them unsuitable for precision circuits where stability is essential. Additionally, device mismatches due to process variations, temperature effects, and voltage drops reduce their accuracy, resulting in mirrored currents that differ from reference values.

Another significant challenge is limited voltage headroom, especially in modern low-voltage technologies. As supply voltages decrease with each generation of semiconductor scaling, simple mirrors struggle to function effectively. These limitations highlight the need for improved topologies, capable of delivering accuracy and efficiency in low-power and low-voltage conditions.

1.4 Introduction to Cascode Current Mirrors

To overcome the drawbacks of simple designs, advanced structures like the cascode current mirror were introduced. By stacking transistors in a cascode configuration, output resistance is significantly increased, reducing the sensitivity of output current to voltage variations. This improves stability and accuracy, making cascode mirrors more suitable for precision applications.

Cascode current mirrors also minimize channel-length modulation effects, a key source of error in MOSFET-based designs. As a result, they achieve better current matching, higher gain, and improved linearity. These advantages make cascode topologies central to modern analog design, particularly in circuits requiring both precision and efficiency.

1.5 Gain in Cascode Current Mirrors

One of the most important benefits of cascode mirrors is their ability to provide high output resistance, which directly translates into higher voltage gain when used as loads in amplifiers. This makes them essential in applications such as operational amplifiers and comparators, where high gain is critical for accurate signal processing.

In addition, higher gain improves the linearity and dynamic range of analog circuits, allowing them to handle a broader range of signals without distortion. Cascode mirrors therefore enhance performance in both low-frequency and high-frequency applications, reinforcing their importance in modern integrated circuit design.

1.6 Noise Considerations

Noise is an inevitable phenomenon in all electronic systems, and in analog circuits, it can severely degrade accuracy. Sources such as thermal noise, flicker noise, and device mismatches introduce unwanted disturbances into signals. For sensitive applications like biomedical instrumentation and communication receivers, reducing noise is a top priority.

Cascode current mirrors are particularly useful in improving noise performance because they stabilize operating points and minimize mismatches. This allows designers to achieve more reliable circuits, ensuring that weak or low-level signals are preserved accurately. Thus, analyzing noise in cascode structures is a critical step in optimizing analog systems.

1.7 Power Consumption in Modern Circuits

In today’s world, power efficiency is just as important as performance. With the rapid growth of portable devices, IoT systems, and wearable electronics, circuits must deliver high accuracy while consuming minimal energy. Cascode current mirrors, though requiring additional transistors, can be designed to strike an effective balance between performance and power.

By providing improved stability and accuracy at low supply voltages, cascode mirrors become ideal for low-power applications. Their role in energy-efficient circuit design ensures their continued relevance as technology scales into more compact and power-constrained systems.

1.8 Applications of Cascode Current Mirrors

Cascode current mirrors are widely used in operational amplifiers, filters, data converters, communication circuits, and biomedical devices. Their ability to deliver high output resistance and stable currents makes them ideal as biasing elements and active loads. For example, in an op-amp, they help achieve high voltage gain, while in ADCs and DACs, they ensure accurate current steering.

In biomedical systems, cascode mirrors provide low-noise, low-power biasing for sensitive amplifiers that measure tiny bio-signals. In communication systems, they stabilize oscillators and mixers, enabling reliable wireless transmission. These diverse applications highlight their role as versatile and indispensable components in analog VLSI.

1.9 Future Trends and Scope

As semiconductor technology continues to scale into the nanometer regime, challenges such as short-channel effects, reduced supply voltages, and variability in device parameters become more pronounced. These changes demand further innovation in current mirror design to maintain accuracy, stability, and efficiency.

Future research may explore digitally assisted analog techniques, adaptive biasing, and hybrid current mirror topologies that combine the strengths of cascode designs with modern digital control. Such innovations will ensure that cascode mirrors remain vital in the evolving landscape of low-power, high-performance VLSI systems.

## Chapter 2: Methodology

2.1 Introduction

The methodology for employing a cascode current mirror revolves around systematically designing, biasing, and optimizing the circuit to achieve maximum gain, efficient power handling, and stable operation under varying power supply conditions. Unlike conventional approaches that focus only on theory, the methodological approach emphasizes stepwise design, simulation, and verification. This ensures that the cascode mirror performs reliably in integrated circuits or discrete analog applications while maintaining accuracy and high output resistance.

2.2 Establishing the Reference Current

The first step in the methodology is selecting a suitable reference current that defines the operation of the mirror. The reference current is typically determined based on the required output current, transistor sizing, and power budget. It is crucial to ensure that the chosen current allows both the input and cascode transistors to remain in the active region during operation. Methodologically, designers calculate the reference current by considering voltage headroom, transistor threshold voltages, and expected load conditions to avoid saturation or cutoff.

2.3 Designing Transistor Parameters

Once the reference current is determined, the next methodological step is transistor sizing. This involves selecting appropriate channel lengths and widths to balance current matching, output resistance, and voltage gain. The input transistor is designed to set the reference current precisely, while the cascode transistor is sized to provide high output resistance and isolate the input transistor from output voltage variations. Methodologically, designers calculate the transistor ratios using formulas for transconductance, output resistance, and the desired gain while considering process limitations and layout constraints.

2.4 Biasing the Cascode Transistor

Proper biasing of the cascode transistor is a critical methodological step to ensure high output resistance and linear operation. The bias voltage is applied to the gate (or base) of the cascode transistor such that it remains in the active region regardless of output voltage swings. Methodologically, this involves calculating the required bias voltage as a sum of the overdrive voltage of the input transistor and the voltage drop across it due to the reference current. Correct biasing ensures that the mirror can maintain accurate current replication under all operating conditions.

2.5 Ensuring Voltage Headroom

Maintaining sufficient voltage headroom is another essential methodological consideration. The total supply voltage must be high enough to accommodate the voltage drops across both the input and cascode transistors without driving either into saturation. Designers calculate the minimum supply voltage by summing the overdrive voltages of both transistors and adding a safety margin for variations in process, temperature, and load. This step ensures that the cascode current mirror operates reliably and that its high gain and output resistance are preserved.

2.6 Simulating Gain and Output Resistance

A key part of the methodology involves simulating the cascode current mirror to verify its gain and output resistance. Simulation is performed under nominal and worst-case conditions to observe the mirror’s behavior when the load varies or the supply voltage fluctuates. Designers extract the small-signal voltage gain and measure output resistance to confirm that the mirror meets design specifications. Methodologically, iterative adjustments to transistor sizing, bias voltage, and reference current are performed until the desired gain and output resistance are achieved.

2.7 Power Optimization Methodology

Optimizing power consumption is critical in the design process. Methodologically, designers calculate the power dissipation across both transistors and distribute it to avoid thermal stress. Techniques such as choosing appropriate transistor sizes, limiting the reference current to the minimum required for accurate operation, and ensuring balanced voltage drops are applied. Simulation under various load currents is used to confirm that the mirror can handle the required power efficiently without exceeding thermal or voltage limits.

2.8 Verifying Supply Voltage Tolerance

The methodology also involves verifying that the cascode mirror performs consistently across a range of supply voltages. Designers test the circuit under lower and higher supply voltages to ensure that current replication remains accurate and that both transistors remain in the active region. This step is critical in ensuring that the mirror can tolerate real-world variations in the power supply without performance degradation. Iterative adjustments to bias voltages and transistor sizing are performed to maximize supply voltage tolerance.

2.9 Final Validation and Iteration

The last step in the methodology is comprehensive validation. This includes checking current matching accuracy, voltage gain, power dissipation, and supply voltage tolerance simultaneously. Any deviations from the design targets are corrected through iterative adjustments in transistor sizing, biasing, or reference current selection. Methodologically, this ensures that the cascode current mirror achieves high gain, efficient power handling, and reliable operation under all anticipated conditions, making it ready for practical circuit integration.

CIRCUIT DIAGRAMS:

## Future Development

Future development in the methodology of cascode current mirrors focuses on enhancing performance, reducing power consumption, and enabling integration with advanced technologies. One promising direction is the scaling of transistors using advanced CMOS and FinFET technologies, which allows higher gain, lower voltage operation, and reduced parasitic effects. Methodologically, this requires revising transistor sizing, biasing schemes, and reference current calculations to exploit the benefits of smaller process nodes while maintaining stability and accuracy.

Another area of development is adaptive biasing techniques, where the cascode transistor’s bias voltage is dynamically adjusted based on supply variations, temperature, or load conditions. This methodology allows the mirror to maintain accurate current replication under real-world fluctuations and improves efficiency in low-power and battery-operated circuits.

Integration with low-voltage and ultra-low-power designs is also a key focus. Methodological innovations include reducing voltage headroom requirements, optimizing transistor stacking, and exploring alternative current mirror topologies (such as regulated cascode mirrors) that maintain high output resistance while operating at lower supply voltages.

Finally, AI-assisted design optimization and automated simulation methodologies are emerging, enabling faster iteration, precise tuning of gain, current accuracy, and power handling. Designers can now leverage computational methods to predict optimal transistor parameters, biasing conditions, and power supply limits, significantly improving the development workflow for high-performance cascode current mirrors.

These future developments aim to make cascode current mirrors more robust, power-efficient, and adaptable, extending their applicability in next-generation analog and mixed-signal circuits.

## References

[1] H. Xie, Z. Wang, G. Liu, J. Lu, and X. Yi, “A novel active-input cascode current mirror with high precision and low power dissipation,” Engineering Reports, vol. 3, no. 1, 2021.

[2] W. Jendernalik, J. Jakusz, R. P. Piotrowski, G. Blakiewicz, and S. Szczepański, “Unity-gain zero-offset CMOS buffer with improved feedforward path,” Electronics, vol. 10, no. 13, pp. 1–15, 2021.

[3] B. Aggarwal, M. Gupta, and A. Gupta, “A comparative study of various current mirror configurations: Topologies and characteristics,” Microelectronics, vol. 53, pp. 134–155, 2016.

[4] G. S. Deo, J. A. Totlani, K. E. Mamidi, and C. V. Mahamuni, “Performance analysis of BiMOS differential pair with active load, Wilson and Widlar current mirrors, and diode connected topology,” Proc. ICICCS, pp. 99–104, 2020.

[5] U. M. Iqbal, L. S. Mahmood, L. Nguyen, and L. Albasha, “Folded cascode current mirror design using Cadence,” Proc. ICCSPA, Sharjah, UAE, pp. 1–4, 2019.

[6] Y. Li, Y. L. Li, and S. M. Yu, “Design optimization of a current mirror amplifier integrated circuit using a computational statistics technique,” Mathematics and Computers in Simulation, vol. 79, pp. 1165–1177, 2008.

[7] N. P. M. Kumar and N. V. R. V. Kumar, “A low voltage very high impedance current mirror circuit and its application,” Proc. ICCC, pp. 1–6, 2013.

[8] B. Razavi, Design of Analog CMOS Integrated Circuits, 2nd ed., New York: McGraw-Hill, 2017.

[9] P. R. Gray, P. J. Hurst, S. H. Lewis, and R. G. Meyer, Analysis and Design of Analog Integrated Circuits, 5th ed., Wiley, 2009.

[10] K. R. Laker and W. M. C. Sansen, Design of Analog Integrated Circuits and Systems, McGraw-Hill, 1994.

[11] Y. Tsividis and C. McAndrew, Operation and Modeling of the MOS Transistor, 4th ed., Oxford Univ. Press, 2011.

[12] R. J. Baker, CMOS: Circuit Design, Layout, and Simulation, 4th ed., Wiley-IEEE Press, 2019.

[13] A. Sedra and K. C. Smith, Microelectronic Circuits, 8th ed., Oxford Univ. Press, 2020.

[14] A. Bellaouar and M. I. Elmasry, Low-Power Digital VLSI Design: Circuits and Systems, Springer, 1995.

[15] M. Alioto, “Ultra-low power VLSI circuit design demystified: Fundamentals and principles of CMOS and beyond,” Springer Series in Advanced Microelectronics, 2019.

[16] S. M. Kang and Y. Leblebici, CMOS Digital Integrated Circuits: Analysis and Design, 4th ed., McGraw-Hill, 2014.

[17] A. S. Sedra, K. C. Smith, and T. C. Carusone, Microelectronic Circuits: Theory and Applications, Oxford Univ. Press, 2019.

[18] A. Allam, M. Dessouky, and A. Amer, “A high swing cascode current mirror with improved gain and reduced power,” IET Circuits, Devices & Systems, vol. 12, no. 3, pp. 211–218, 2018.

[19] J. Ramirez-Angulo, R. G. Carvajal, and A. Lopez-Martin, “Low-voltage low-power analog circuit design techniques,” IEEE Circuits and Systems Magazine, vol. 6, no. 4, pp. 32–43, 2006.

[20] I. Maity and A. Majumdar, “Computation of key analog circuit parameters employing different topological configuration of current mirror circuits: A comprehensive analysis of gain, noise, and power consumption,” RP Current Trends in Engineering and Technology, vol. 3, no. 2, pp. 23–29, 2024.

[21] A. B. Bhattacharya and S. Banerjee, “Design and performance evaluation of CMOS current mirror configurations,” International Journal of Electronics and Communication Engineering, vol. 12, no. 4, pp. 55–63, 2019.

[22] S. Maheshwari, “High performance cascode current mirror for analog IC design,” Journal of Active and Passive Electronic Devices, vol. 11, no. 2, pp. 87–94, 2016.

[23] A. Lopez-Martin, R. G. Carvajal, J. Ramirez-Angulo, and J. Tombs, “Low-voltage super class-AB CMOS current mirror,” IEEE Journal of Solid-State Circuits, vol. 40, no. 3, pp. 526–530, Mar. 2005.

[24] C. A. De La Cruz-Blas, A. Lopez-Martin, and R. G. Carvajal, “Low-voltage low-power high-swing cascode current mirror,” Analog Integrated Circuits and Signal Processing, vol. 72, no. 2, pp. 391–396, 2012.

[25] P. Jespers and B. Murmann, Systematic Design of Analog CMOS Circuits: Using Pre-Computed Lookup Tables, Cambridge University Press, 2017.

[26] A. Elwakil and K. N. Salama, “Low-power analog circuit design in nanoscale CMOS: A review,” IEEE Circuits and Systems Magazine, vol. 14, no. 3, pp. 45–61, 2014.

[27] H. J. Oguey and D. Aebischer, “CMOS current mirrors for low-voltage precision applications,” IEEE Transactions on Circuits and Systems II: Analog and Digital Signal Processing, vol. 45, no. 7, pp. 703–708, Jul. 1998.

[28] A. H. M. Elwan and A. M. Soliman, “Low-voltage low-power current mirror using bulk-driven MOS transistors,” Electronics Letters, vol. 33, no. 17, pp. 1420–1421, 1997.

[29] S. Pandey and A. K. Singh, “Performance comparison of simple, Wilson, and cascode current mirrors using 90nm CMOS technology,” Proc. International Conference on Emerging Trends in Engineering and Technology (ICETET), pp. 55–59, 2020.

[30] R. T. Rajeswaran, G. Anand, and S. Nirmal, “Noise and power trade-offs in cascode current mirrors for biomedical instrumentation,” Biomedical Signal Processing and Control, vol. 78, p. 103898, 2022.